# Experiment 11.1: IIR Low-Pass Filter Implementation on TMS320C5416 DSP Processor (Hardware)

## Aim

To implement a second-order IIR (Butterworth) low-pass digital filter in TMS320C54xx assembly language, interfacing the ADC and DAC on the TMS320C5416 DSP starter kit, for a cutoff frequency of 4 kHz at a sampling frequency of 62 kHz.

## MATLAB Code

;**************************************************************************
; LOW PASS FILTER (IIR)
;**************************************************************************
; Butterworth filter, IIR Filter, Created by Filter Design Package,
; (c) 2005, Vi Microsystems Pvt. Ltd.
; Approximation Type : Butterworth
; Filter Type        : Low Pass Filter
; Filter Order       : 2
; Sampling Frequency : 62 kHz
; Cutoff Frequency   : 4.000000 kHz
 
; Variable declarations
XN   .SET 10H
XNM1 .SET 11H
XNM2 .SET 12H
YN   .SET 13H
YNM1 .SET 14H
YNM2 .SET 15H
SUM  .SET 16H
 
; Filter coefficients
A1 .SET 0E903H
A2 .SET 0905H
B0 .SET 082H
B1 .SET 0104H
B2 .SET 082H
 
    .mmregs
    .text
START:
    STM  #0140H,ST0      ; data page initialization
    RSBX CPL             ; reset compiler mode bit
    RSBX FRCT            ; reset fractional mode bit
    NOP
    NOP
    NOP
 
    ST #0H,XN
    ST #0H,XNM1
    ST #0H,XNM2
    ST #0H,YN
    ST #0H,YNM1
    ST #0H,YNM2
 
REPEAT:
    PORTR 6,0            ; start ADC conversion
    NOP
    NOP
    NOP
    NOP
    PORTR 4,0            ; read sampled data
    LD   0,A
    AND  #0FFFH,A        ; extract 12-bit sample
    XOR  #0800H,A        ; correct 2's complement
    SUB  #0800H,A        ; convert to processor format
    STL  A,XN
 
    LD  #0H,B
    LD  XN,A
    STLM A,T
    MPY #B0,A
    LD  A,B
 
    LD  XNM1,A
    STLM A,T
    MPY #B1,A
    ADD A,B
 
    LD  XNM2,A
    STLM A,T
    MPY #B2,A
    ADD A,B
 
    LD  YNM1,A
    STLM A,T
    MPY #A1,A
    SUB A,B
 
    LD  YNM2,A
    STLM A,T
    MPY #A2,A
    SUB A,B
 
    SFTL B,-12            ; right-shift result by 12 bits
    STL  B,YN
 
    LD  YNM1,A            ; shift delay-line registers
    STL A,YNM2
    LD  YN,A
    STL A,YNM1
    LD  XNM1,A
    STL A,XNM2
    LD  XN,A
    STL A,XNM1
 
    LD  YN,A
    ADD #800H,A           ; add DC offset
    STL A,YN
    PORTW YN,04H          ; output to DAC
    B REPEAT

## Output

No output screenshots were available for this hardware-based experiment.

## Result

The assembly-coded second-order Butterworth IIR filter was implemented and interfaced with the TMS320C5416 DSK's on-board ADC/DAC. Hardware execution filters the sampled analog input at a 4 kHz cutoff (62 kHz sampling rate); no output waveform capture was available for this hardware trial.