# Experiment 7.2: Windowing Effects on Spectral Estimation (Rectangular / Hamming / Hanning)

## Aim

To study the effect of Rectangular, Hamming and Hanning windows on the spectral estimation of two closely-spaced sinusoids, comparing main-lobe width, sidelobe levels and frequency-resolution performance.

## MATLAB Code

clc; clear; close all;
% Signal
fs = 1000; N = 256; t = (0:N-1)/fs;
x = sin(2*pi*100*t) + 0.8*sin(2*pi*110*t);
NFFT = 2048; f = (0:NFFT/2-1)*(fs/NFFT);
% Windows
w = {rectwin(N), hamming(N), hann(N)};
names = {'Rectangular','Hamming','Hanning'};
% Processing
for i = 1:3
 xw = x(:).*w{i};
 X = fft(xw,NFFT);
 P{i} = abs(X(1:NFFT/2));
 P{i} = P{i}/max(P{i});
end
%% Individual Spectrums
figure;
for i = 1:3
 subplot(3,1,i);
 plot(f,20*log10(P{i}),'LineWidth',1.5);
 title(['Spectrum using ' names{i} ' Window']);
 xlim([0 250]); grid on;
end
%% Comparison
figure;
plot(f,20*log10(P{1}),'k',f,20*log10(P{2}),'r',f,20*log10(P{3}),'b','LineWidth',1.5);
legend(names); title('Window Comparison'); xlim([50 170]); grid on;
%% Window Shapes
figure;
for i = 1:3
 subplot(3,1,i); plot(w{i},'LineWidth',1.5);
 title([names{i} ' Window']); grid on;
end
%% Peak Detection
for i = 1:3
 [pks{i},loc{i}] = findpeaks(P{i},f,'MinPeakHeight',0.3);
 fprintf('\n%s Window Frequencies:\n',names{i});
 disp(loc{i});
end
%% Peak Marking (Hamming)
figure;
plot(f,20*log10(P{2}),'LineWidth',1.5); hold on;
plot(loc{2},20*log10(pks{2}),'ro','MarkerFaceColor','r');
title('Detected Frequencies (Hamming)'); xlim([50 170]); grid on;
%% Main Peaks
fprintf('\nMain Spectral Peaks:\n');
for i = 1:3
 [~,idx] = max(P{i});
 fprintf('%s Window Peak = %.2f Hz\n',names{i},f(idx));
end

## Output / Screenshots

Fig. 7.2(a): Spectrum of the two closely-spaced tones under each window

Fig. 7.2(b): Overlaid window comparison near the two tones

Fig. 7.2(c): Shapes of the Rectangular, Hamming and Hanning windows

Fig. 7.2(d): Detected spectral peaks (Hamming window) at 100 Hz and 110 Hz

## Result

The Rectangular window resolves the two closely-spaced tones (100 Hz and 110 Hz) with the narrowest main lobe but the highest sidelobes; the Hamming and Hanning windows suppress sidelobes progressively more but widen the main lobe, illustrating the fundamental resolution–leakage trade-off in spectral estimation.