# Experiment 8.2: Polyphase Implementation of Decimation & Interpolation Filters

## Aim

To implement polyphase decimation and interpolation of a two-tone test signal using an FIR low-pass filter with the upfirdn function, and to compare the reconstructed signal against the original.

## MATLAB Code

clc; clear; close all;
%% Input Signal
Fs = 1000; % Sampling Frequency
t = 0:1/Fs:1; % Time Vector
% Test Signal
x = sin(2*pi*50*t) + 0.5*sin(2*pi*120*t);
%% FIR Low-Pass Filter Design
N = 20; % Filter Order
Fc = 0.4; % Normalized Cutoff Frequency
h = fir1(N,Fc,hamming(N+1));
%% Decimation Factor
M = 2;
% Polyphase Decimation
y_dec = upfirdn(x,h,1,M);
%% Interpolation Factor
L = 2;
% Polyphase Interpolation
y_int = upfirdn(y_dec,h,L,1);
%% Adjust Length for Comparison
if length(y_int) > length(x)
 y_int = y_int(1:length(x));
elseif length(y_int) < length(x)
 x = x(1:length(y_int));
end
%% Time Axes
t1 = (0:length(x)-1)/Fs;
t2 = (0:length(y_dec)-1)/(Fs/M);
t3 = (0:length(y_int)-1)/Fs;
%% MAIN GRAPH
figure('Name','Polyphase Decimation and Interpolation');
subplot(4,1,1)
plot(t1,x,'b','LineWidth',1.5)
title('Original Input Signal'); grid on;
subplot(4,1,2)
stem(t2,y_dec,'r','filled')
title('Decimated Signal (M = 2)'); grid on;
subplot(4,1,3)
plot(t3,y_int,'k','LineWidth',1.5)
title('Interpolated/Reconstructed Signal (L = 2)'); grid on;
subplot(4,1,4)
plot(t1,x,'b','LineWidth',1.5); hold on
plot(t3,y_int,'r--','LineWidth',1.5)
title('Comparison of Original and Reconstructed Signals')
legend('Original Signal','Reconstructed Signal'); grid on;
%% Display Information
fprintf('Original Signal Length = %d\n',length(x));
fprintf('Decimated Signal Length = %d\n',length(y_dec));
fprintf('Interpolated Signal Length = %d\n',length(y_int));
%% Separate Figure
figure;
plot(t1,x,'b','LineWidth',2); hold on
plot(t3,y_int,'r--','LineWidth',2)
title('Original vs Reconstructed Signal')
legend('Original Signal','Reconstructed Signal'); grid on;

## Output / Screenshots

Fig. 8.2(a): Original, decimated, interpolated and comparison signals

Fig. 8.2(b): Original vs reconstructed signal (overlaid)

## Result

The polyphase decimation–interpolation chain (M = L = 2) reconstructs a scaled, filtered approximation of the original two-tone signal; some amplitude attenuation and smoothing is visible because the FIR filter removes energy near and beyond the cutoff frequency during the resampling process.